# AutoTherm Research — R_th Reduction Strategy Notes

Compiled: May 12, 2026  |  Project: autotherm-research (DGX Spark Cluster)

# 1. Background — How R_th is Computed

The simulation uses a simplified proxy model (not rigorous NEMD). prepare.py runs a 3-component Lennard-Jones fluid in GROMACS and extracts cross-component interaction energies to estimate thermal resistance:

thermal_coupling  =  -(E_AB + E_AC + E_BC) / box_area_nm2

R_th              =  k_model / thermal_coupling      (k_model = 1.38e-2)

Key insight: to lower R_th, maximise thermal_coupling — i.e. strengthen the LJ attraction between all three component pairs (metal-filler, metal-binder, filler-binder).

# 2. Best Results Found Across All Runs

| Git Commit | R_th (m2K/W) | metal/filler/binder | Temp (K) | N_TOTAL | cross-eps (mf/mb/fb) |
| --- | --- | --- | --- | --- | --- |
| d8aca7d | 4.552e-06 BEST EVER | 53% / 32% / 15% | 350 | 16,000 | 7.0 / 6.0 / 5.0 |
| 3b317f9 | 7.623e-06 | 55% / 35% / 10% | 400 | 30,000 | 5.5 / 3.0 / 2.5 |
| 16fab86 | 7.974e-06 | 55% / 30% / 15% | 300 | 40,000 | 5.5 / 3.0 / 2.5 |

# 3. Key Levers for Reducing R_th

## 3.1  Cross-interaction Epsilons — Highest Impact

The single biggest driver identified. The best result (d8aca7d, R=4.552e-06) used cross-epsilon values of 7.0 / 6.0 / 5.0 kJ/mol — close to the physical ceiling of 8.0 kJ/mol. Recent runs settled at 5.5 / 3.0 / 2.5 and never beat the old result. The program.md example only suggested "try 5.0-6.0" which caused the LLM to be too conservative.

metal-filler epsilon: push toward 8.0 — most important pair (both components have high intrinsic conductivity)

metal-binder epsilon: push toward 7.0-8.0 (models surface functionalisation with silane coupling agents)

filler-binder epsilon: push toward 6.0-7.0

Try all three simultaneously near ceiling: 7.5 / 7.0 / 6.5 as next experiment

## 3.2  Aggressive Target in program.md

The run that found the best result had a target of < 2.0e-8 m2K/W in program.md. The current program.md uses < 4.0e-6. The aggressive target forces the LLM to push parameters toward physical limits rather than settling for incremental gains. With an easy target the LLM sees 7.97e-6 and thinks "try a bit harder"; with 2.0e-8 it knows it must push everything to the ceiling.

Action: restore target to < 1.0e-7 or even < 1.0e-8.

## 3.3  N_TOTAL — Two Competing Effects

N_TOTAL affects results in two distinct ways:

Statistical accuracy (valid): more atoms gives less noise and better phase-space sampling. Real benefit from N=500 up to roughly N=10000-16000; diminishing returns above that.

Formula artifact: BOX_AREA scales as N^(2/3) while total LJ energy scales as N, so thermal_coupling proportional to N^(1/3). Larger N gives structurally lower R_th regardless of composition. Doubling N reduces R_th by ~1.26x as a pure size effect, not a material improvement.

Evidence: N=40000 with low cross-eps gave 7.97e-6 (worse than N=16000 with high cross-eps at 4.55e-6). Composition dominates, but size effect is real and pollutes comparisons across runs.

Actions / options:

Option A — Fix N_TOTAL to a constant (e.g. 8000-12000) across all iterations so R_th values are directly comparable.

Option B — Fix the formula in prepare.py: normalise by N_TOTAL instead of BOX_AREA to get a size-independent quantity: thermal_coupling = -total_cross / N_TOTAL

Note: Option B invalidates all historical R_th values and requires a new baseline run.

Recommendation: Fix N_TOTAL=8000 in program.md for now (fast + comparable); fix formula later.

## 3.4  Warm-Starting from Best Known Composition

Currently each run resets structure.py to the original baseline and the Bayesian optimiser starts from scratch, rediscovering known good regions. The best-ever composition (d8aca7d: git commit on cluster) could be used as the new baseline so the LLM explores perturbations around that known good region.

Action: copy d8aca7d structure.py to structure.py.baseline before next run.

Risk: reduces exploration diversity — may miss other good regions of parameter space.

Mitigation: run one warm-start run + one cold-start run in parallel on different nodes.

## 3.5  Individual LJ Epsilons (Per Component)

In addition to cross-interactions, the per-component epsilons (metal, filler, binder) affect self-cohesion and packing density. Best result used metal eps=4.20, filler eps=2.80, binder eps=1.60 — none were pushed to the ceiling (8.0). Higher individual eps = denser packing = more cross-interface contact area.

Action: try metal eps=6.0-7.0, filler eps=5.0-6.0 combined with high cross-eps.

## 3.6  Composition Fractions

Best result: metal=53%, filler=32%, binder=15%. Filler fraction of 32% is in the optimal percolation range (20-40%). Metal-heavy compositions work well with InGa as the continuous matrix. Low binder (10-15%) keeps thermal resistance paths short.

Action: explore metal 50-60%, filler 28-38%, binder 8-15% around best known point.

## 3.7  Temperature

Best result used T=350 K. Recent runs tried 300 K and 400 K with worse results. 350 K may be a sweet spot balancing liquid-metal density (favours lower T) vs binder chain mobility (favours higher T).

Action: fix T=350 K for next few runs to isolate composition effects.

Later: fine-sweep 320-380 K once composition is tuned.

## 3.8  SIM_TIME_PS

Best result used SIM_TIME_PS=100 ps. Longer runs give more accurate energy averages but cost more wall-clock time. Recent runs used 120-200 ps. For initial exploration, 100 ps is sufficient; use 200-300 ps only for confirming a promising composition.

# 4. Suggested Configuration for Next Run

## program.md changes:

Set target to < 1.0e-8 m2K/W (aggressive — forces LLM to push to ceiling)

Cap N_TOTAL at 8000-12000 for fast iterations and fair comparisons

Add explicit note: best result used cross-eps = 7.0/6.0/5.0 — explore 7.0-8.0 range for all pairs

Add example: "try metal-filler eps=7.5, metal-binder eps=7.0, filler-binder eps=6.5"

Reduce SIM_TIME_PS suggestion to 80-120 ps for exploration phase

## structure.py.baseline:

Consider warm-starting from d8aca7d composition (metal=0.53, filler=0.32, binder=0.15, T=350K, N=16000).

## prepare.py (longer term):

Fix thermal_coupling normalisation to be size-independent so N_TOTAL can be freely varied.

# 5. Open Questions

Is the formula size-artifact intentional (larger box = thicker TIM layer = physically valid) or a model flaw?

What is the theoretical lower bound on R_th for this 3-component LJ system when all epsilons are at max (8.0)?

Should N_TOTAL and SIM_TIME_PS be fixed constants so the optimiser focuses only on composition + LJ params?

Would multi-node parallel runs with different seeds on the same composition help confirm statistical significance?

Does the sigma parameter for cross-interactions have as much impact as epsilon? (not yet explored systematically)

Could a 4-component system (e.g. adding a surface-active agent) improve performance further?